 Survey
KW5.0 Air heater (hereinafter referred to as the heater) is independent to the original engine system, it makes use of 12V or 24V direct current to drive. There are two kinds of control mode of  the heater: Automatic control mode and Manual control mode. The heater adopts light diesel which corresponds to the environmental temperature as fuel, and it can be started and operated normally at the temperature of above -40℃. The inhaled fresh air is heated to hot air through heat exchanger by the energy which comes from fuel burning, then blown to where it is needed. 

Advantage：
5kw auto parking heater owns the advantage of compact structure, light weight, high thermal efficiency, economize on electricity and fuel, easy installation.
Technical specification：

	Power（W）
	5000

3000

5000

3000

5000

5500

	Heating medium
	Air

	Fuel
	Diesel

	Fuel consumption（l/h）
	0.18-0.48

	Rated voltage（V）
	12V / 24V

	Working temperature（℃）
	-50℃—45℃

	Weight（KG）
	5.2

	Size（mm）
	380x145x177


Application field of Air heater

5kw air parking heater is not affected by the engine ,and it is supplied for the following vehicles with corresponding power.

· All kinds of auto and trailers.

· Construction machinery

· Agricultural machinery

· Boat, ship, yacht

· Caravan 

Main heater installation

The main heater could be installed both inside and outside of the vehicle. If the heater is installed outside the vehicle, measures must be taken to avoid splashing water onto the heater. Enough space must be provided for installation for the convenience of heating air flow and installation,maintenance of the main heater.

Installation of Electrical System

[image: image1.png]PT1000

Case temperature

NT100K

inlet air temperature

2-core socket

controller

fio

blower U

g

I/m;el pumi

e
e D
e

W
[N

ON switch

o chynight light switfh

vce

ON

Car batfery positive poIeI

Car battpry negative pole

controllel
3 4 i
A aan
RB7 9 10 1xd
11 12

sodket wire connection finner wire connection





Operation and Control

After the installation, the remote control 5kw air parking heater shall be turned on repeatedly for a few times to make the fuel tube full-filled, so as to avoid staring failure due to lacking fuel.

Controller

[image: image2.jpg]



1--Controller knob; 2--Manual Switch; 3--Automatic switch; 4--Power off / Function; 5--LCD; 6-- Fahrenheit; 7--Centigrade; 8--Temperature / Power percentage;

9--Mode
Maintenance
During the running of air parking heater, it tests and checks the operating state and fault in the whole course, and the controller shows fault codes on the LCD constantly. 

	Fault Code(ER)
	Implication
	Solution

	01
	Power failure
	Check the power connection

	02
	Motor failure
	Check the motor connection and control panel

	03
	Fuel pump failure
	Check the plug-in unit connection and control panel

	05
	Flame  extinguishing
	Check the fuel circuit

	07
	Ignitor failure
	Check the plug and control panel

	08
	Failure of second start
	Check the flame state in combustion chamber, fuel circuit and voltage

	09
	Failure of intake air sensor
	Check the plug and control      panel

	11
	Failure of cover sensor
	Check the plug and control panel

	12
	Failure of communication(Connection broken between controller and main control panel)
	Check if any looseness,open  circuit or misconnection on the data lines and plug-in components


[image: image3.jpg]


Product Disply Pictures
[image: image4.jpg]



